Exercise 3: Develop a pure Java Card applet

(Electronic purse)

Description

In this exercise you will create a simple Electronic Purse applet using standard Java Card 2.0 APDU handling.

Goals

After completing this exercise, you will be able to:

- Implement the install() method.

- Register the applet.

- Incorporate Java Card 2.0 APDU handling code into your applets.

- Execute the APDU exchange feature of GemXpresso.

Phase I – Implementing the Purse

- Register the applet in the install() method.

- In the process methods, verify that the APDU is of the correct class and throw the appropriate exception if it is not.

- In the process method, determine the instruction being sent and then branch to the appropriate method.

- In the debit() and credit() methods, retrieve the data from the APDU buffer.

- In the getBalance() method, send the balance.

- Compile Purse applet successfully

Phase II – Loading the applet

- Rebuild applet in GemXpresso

- Load Purse applet onto card

- Select Purse applet

Phase III – APDU exchange

- Select Card -> APDU exchange

- Execute each of the 3 commands (INS_CREDIT, INS_DEBIT, INS_GETBALANCE)

- Try and use the macro functionalities for each command sent to the card